Structuring the Photodiode chapter.

Introduction:

- Set the scene: cryogenic -> 2um -> extended InGaAs + InSb? N.B. avoid tunnel vision, mention MCT.

- Limitation in previous work was the quality of the PD?

- Are there operating conditions which would be suitable for GW detection?

What is an extended InGaAs photodiode:

- PIN junction that converts light to electrons.

- Problems with the lattice etc.

- List of photodiodes that were tested to see if this problem exists in commercial PDs

‘What is needed from a photodiode in a gw detector?’ section:

- Sensitive to 2um (obviously).

- High quantum efficiency. (thickness and AR coating)

- Low dark current/noise. (lattice quality)

- Handle ~10mW of light i.e. linear

This means we have to do <list of  measurements that will be mentioned in chapter>

Dark noise experiment + result and discussion

- Room temperature

- Cooled

- Low Bias noise measurement

QE experiment + result and discussion

- QE was measured for diodes that were linear…

- Shot noise measurement: real increase in QE.

Linearity/Saturation experiment + result and discussion

- AC to DC ratio measurement.

Discussion of, where relevant, results in context with each other:

- High QE needs high bias

- Linearity needs high bias

- But high bias leads to noise

- Table that sums up goodness of photodiodes. Explain overall trends.

Conclusion

- To fix this problem, InGaAs needs to be grown slowly to avoid lattice defects which give rise to 1/f noise. Commercially available photodiodes are not made this way, it would take a large order to try this. A manufacturer has proposed to commission of wafers.

Intro bit

Sign posts. Basic introduction of what is important and what was done

8.1

QE req.

Light handling req.

Noise req.

Discussion of why heterodyne wont work.

8.2

Sign post and basic intro.

PN diode explanation

Photon absorption in diode + depletion region

Widen depletion region with bias

Widen depletion region with intrinsic layer

What ingaas is.

Making ingaas extended

Defects in extended InGaAs that is thick enough to have good qe.

What is a defect.

AR coating.

Dark noise and dark current.

Noise formula.

8.3

Dark noise was tested in commercially available PDS. (repeated?). What was measured in each PD.

8.4

What was measured + sign posts to method and main result.

8.4.1

The two measurement styles

Why shot noise needs to be measured, and what you need to do this.

TIA description

optics and laser description

Beam parameter description

How the noise was measured.

Calibration of measurements with 1111 kHz peak.

8.4.2

What effect increasing the bias had.

Shot noise measurement at 0.6V.

Saturation at 0.6V.

Measurement of ‘absolute qe’

8.5

Why dark noise was measured? To find balance between qe and noise. Sign post to QE increase. Signpost and repeat of the benchmark

Method  (PD circuit) and sign post to results.

8.5.1

Results.

Duplicate photodiodes and batch variation

8.5.2

The range of temperatures chosen and how the diodes were cooled.

What the results show.

8.6

Laser components photodiodes saturate.

8.7

IG24 exceeds shot noise by x2. Sign post to method.

Noise projection to rule out bias noise

Ideality factor.

Result: no idea whats going on.

8.8

InSb intro. What was measured.

How it was measured.

The effect of using a reverse bias.

8.9

Repeat of information about low qe. Say that the data found supports this.

Limitation of the results -> variance between batch is unknown.

Repetition of result of QE, discussion of why qe improves.

Repetition of Laser components result, discuss possible ways to avoid low power handling.

Discussion of PD circuit.

Discussion of dark current and dark noise

Discussion of cooled results.

8.10

Conclusion/summary.